Video Conference:  Help sheet: 
Refer also to the help sheet for your remote

Main set up for View Station:

When you enter the Video Conference [VC] room the VC unit and TVs should be switched on [they should remain switched on to assist asnet in remote monitoring of the unit and the address book]

Pick up the remote and the camera should ‘wake up’ and you will see the screen below. This is the Home page and can be accessed by clicking the ‘home’ or ‘menu’ buttons

Homepage for the VC unit – address book shows the yellow selection border

The Arrow keys: on the remote allow you to move the yellow selection border [in menu mode]

and to move the TV top camera around [in Near camera mode]

Set up: before joining in a VC session make sure you are comfortable and set the unit up so you are in shot, and can switch to document camera or other shots you might want as part of the conversation.

If there are no presets set for the document camera and DVD/Video player use the Near button to bring the icons for these up on the TV screen [as below]

Move your shot to these using either the number or the arrow keys to move the yellow selection border [this has to be done quite quickly]

Dialling in:

A point to point call is where you dial to just one other site.

To do this:

- Go to the home page [by selecting home or menu button]

- Select the address book [using your arrow keys to move the yellow selection border]

The next screen you see may be this one below, [it shows the last six sites dialled] if the school you want is not on here go to the Address book

This will be in the bottom right of the above screen click on the selected icon to access the screen you see below

At the moment the selection is at the top, use the arrow key to scroll down through the list to find the school you want to dial. You can use the tabs out the side to move more quickly through the list. When you have the site you want dial by selecting that entry [with the centre select button]

NB: if allowed you can use the FAR button to move the camera at the other site

VC Classes

To access your class sessions you will be making a Bridge or conference call. This is where several sites all dial into the same room on the Ministry Bridge.

NB: In this instance you can’t use the Far button the Bridge acts as the controller

In a bridge call there can be several sites all connected to the one room [in the same way you can conference call on the phone]. In this instance the Bridge itself acts as ‘the camera’ and the person/school on screen get there by activating the camera with sound [generally by speaking].

Sometimes it is difficult to know who wants to speak next and there is a slight delay – which you will not get used to [the eTeacher will direct questions to various sites to allow everyone a chance to speak.] Generally if you start speaking carry on and finish your sentence.

In a bridge call everyone sees the one who is talking, eg site A

- when site B speaks, the bridge switches to them

- Now everyone sees site B [except site B - who appear to be addressing site A] this is useful to remember when you have asked your questions then muted your mic’

Participants at the Languages VC – what they are projecting shows on the left screen

Everyone is seeing Cheryl from WHS who is speaking, [site B]
 Cheryl  sees Sarah Perkins the Languages Advisor who just asked her a question [site A]

The importance of muting your microphone between comments or questions is to prevent the Bridge/camera inadvertently switching to you when you rustle papers, sneeze or pick up your pencil or the lawn mower goes past the window outside.

It is also vital to mute before moving your microphone, as it is very sensitive [remember: red button=mute].

Your eTeacher will inform you of the microphone set up for their class.

To join your class VC or other Bridge VC events

When you are ready to make your bridge call

- click the home/menu button and go to speed dial or address book

- select 0600CLASS – the VC unit will start dialling

The next screen you will see is the welcome screen to NZEdNet and you will be asked for a PIN number [your teacher will have given you eg. 1234#]

In order to enter the PIN you need to bring up the number pad.

Click the * star key on your remote and enter the PIN [sometimes it requires a couple of attempts and at a measured pace to be accepted – don’t forget the # at the end]

Next you will be asked to record your name, this is followed very quickly by the tone and the length of time you are given is also short [don’t forget to unmute]

If you miss it or have only a cut off sound you can push 1 to re-record otherwise just wait to join the conference.

To exit the conference hit the green call hang up button twice.

Trouble Shooting:

Camera is moving/light is on but TVs show nothing

- Are the TVs switched on?

- Are the TVs on the correct channel? [use the button that is a circle and arrow to switch to the right channel]

- Check all the cables connected to the VC unit there is a plan for this in your school [it should be somewhere near the VC unit]

Can see but don’t appear to be heard?

- Is the red light showing on the microphone?

- red=mute mute also will show as a red crossed circle in the lower left of the main TV screen [as below]

Only seeing a blue screen?

- You are most likely projecting the document camera or DVD/video player and they are switched off

- Try changing to TV top camera by clicking the ‘near’ button and switching between the icons that appear on screen [or try a preset]

The bridge keeps telling me the PIN I am entering is invalid

  - Check you have the number correct

  - Try again with the mute button on

  - Try putting in the numbers in a rhythm [1, 2, 3, 4] smoothly

There is no number pad showing but I hear beeping?

  - The number pad is active but invisible enter the PIN and press # as usual

  - If problem persists try disconnecting and begin the dial process again

Unable to dial a number or the address book is empty

- Try switching off the TV top camera counting to 30 then switching it on again [it should start up with the Polycom icon and a cinematic sound]

- Watch the icons just above your IP address [lower centre screen]. They should register ticks and go green [if orange or red ring asnet]

- Ring asnet 0800 163 525 [they should be able to see remotely if there is a problem and/or isolate if it is the unit or the local network

Ask for help from the eSchool Coordinator, eDean or your school VC person

Important Help numbers:

asnet: 0800 163 525

SchoolZone: 0800 755 966